Assignment questions on 4th unit

- Explain the cache optimization: Giving Read priority over write priority.

- Consider the following  code sequence.

STR   R3,  512 (R0)

LDR   R1,  1024 ( R0)

LDR  R2,  512 (R0)

Assume Direct mapped: Write –through cache that maps 512 and 1024 to the same block. Four word write buffer that is not checked on a read miss. Will the value in R2 always be equal to the value in R3?  Explain with steps.

- Explain write through cache and write back cache with respect to giving read priority over write priority.

- Explain virtual address. How it differs with physical address.

- How virtual address is translated to physical address. Explain with suitable diagram.

- What are the two tasks in address translation? Explain.